# 05 — Client Core & UI

# 05 — Client Core & UI

Revision: 1 Last modified: 2026-07-05T14:20:00Z Status: Draft — consolidated from 03-client-core-and-ui.md and v04-client/*.

## 1. Position

This section covers the stack from the Rust helix-core FFI surface up through the Flutter helix-ui apps and the per-platform tunnel shims.

The client stack consumes:

- Data-plane contracts from 01-data-plane.md (Transport trait, TunnelStatus, RouteMap).

- Control-plane contracts from 02-control-plane.md (REST/WS, WatchNetworkMap, enrollment).

It owns everything from the FFI surface upward.

## 2. Shared-codebase strategy

| Layer | Shared by | Technology |
| --- | --- | --- |
| Rust core (helix-core) | Access, Connector, Gateway edge | Rust — WG, transport, reconciler, kill-switch |
| Flutter UI (helix-ui) | Access, Connector, Console | Dart/Flutter — one tree, three flavors |
| Per-platform shim | Access/Connector per OS | Swift/Kotlin/C++/Qt — creates TUN, hands fd to core |

Console is API-client only — it links no Rust tunnel core.

## 3. helix-core Rust workspace

helix-core/
├── crates/
│   ├── helix-transport/   # Transport trait + carriers
│   ├── helix-wg/          # boringtun wrapper
│   ├── helix-tun/         # TUN abstraction
│   ├── helix-route/       # RouteMap, CompiledPolicy
│   ├── helix-daita/       # optional shaping
│   ├── helix-core/        # orchestrator + reconciler + state machines
│   └── helix-ffi/         # FFI surface (staticlib/cdylib)
└── bin/
    └── helixd.rs          # --mode={client|connector} daemon

## 4. FFI surface (helix-ffi)

Dart (via flutter_rust_bridge v2) and native shims (via UniFFI / C-ABI) call:

pub fn helix_start(cfg: ClientConfig) -> Result<(), CoreError>;
pub fn helix_stop() -> Result<(), CoreError>;
pub fn helix_set_exit(selection: ExitSelection) -> Result<(), CoreError>;
pub fn helix_pin_transport(kind: Option<TransportKind>) -> Result<(), CoreError>;
pub fn helix_status_stream(sink: StreamSink<TunnelStatus>);
pub fn helix_set_shields(shields: ShieldConfig) -> Result<(), CoreError>;
pub fn helix_advertise(cidrs: Vec<IpNet>) -> Result<(), CoreError>;
pub fn helix_attach_tun(fd: RawFd) -> Result<(), CoreError>;

TunnelStatus is the hot status stream consumed by the UI:

pub enum TunnelStatus {
    Disconnected,
    Connecting { transport: Option<String> },
    Handshaking,
    Connected { transport: String, rtt_ms: u32 },
    Reconnecting,
    Down { reason: String },
}

## 5. Size and memory build strategy

The iOS Network Extension memory ceiling is the hardest budget:

[profile.release]
opt-level     = "z"
lto           = "fat"
codegen-units = 1
panic         = "abort"
strip         = true

- iOS uses staticlib + build-std with panic-immediate-abort for the smallest footprint.

- panic=abort means no catch_unwind at the FFI boundary; failures are returned as typed CoreError.

- Runtime: bounded packet-buffer pool, no per-flow state growth.

## 6. Flutter UI (helix-ui)

One tree, three flavors via runHelixApp(flavor, home, capabilities):

| Flavor | Capabilities |
| --- | --- |
| Access | core_ffi, tunnel shim, connect UI |
| Connector | core_ffi, advertise/route config, optional slim UI |
| Console | no core_ffi; admin CRUD + live topology |

State management: Riverpod providers; UI is a pure function of the FFI status stream.

## 7. Per-platform tunnel shims

| Platform | Shim | Mechanism |
| --- | --- | --- |
| iOS / macOS | shim-apple | NEPacketTunnelProvider |
| Android | shim-android | VpnService + JNI |
| Windows | shim-windows | wireguard-nt / wintun + privileged service |
| Linux | shim-linux | kernel WG + systemd |
| HarmonyOS NEXT | shim-harmonyos | Network Kit VPN ability + NAPI |
| Aurora OS | shim-aurora | Qt6/QML + C++ tun |

The shim creates the TUN and hands the fd to helix-core via attach_tun(fd).

## 8. Connector mode

The Connector runs the same Rust core with --mode=connector:

- Does not capture the device's default route.

- Advertises served CIDRs.

- NATs/forwards decapsulated packets into the LAN interface.

## 9. Kill-switch + DNS

- Core-owned state machine drives the OS firewall.

- On tunnel drop or transport escalation, plaintext egress is blocked.

- DNS is forced through the tunnel; plaintext :53 off-tunnel is blocked.

## 10. Cross-references

- Detailed FFI → ../../v04-client/ffi-surface.md

- Flutter UI → ../../v04-client/helix-ui-flutter.md

- Core Rust packaging → ../../v04-client/helix-core-rust.md

- Platform shims → ../../v04-client/shim-*.md

- State management → ../../v04-client/state-management.md

- Design system → 10 — Design System

Sources: docs/research/mvp/final/03-client-core-and-ui.md, v04-client/*.md.