# Linux shim (kernel WG / tun)

# Linux shim (kernel WG / tun)

Revision: 1 Last modified: 2026-06-25T00:00:00Z

Volume 4 (Clients) nano-detail specification — deepens the Linux row of the TunnelPlatform shim matrix [03-client §5.4]. SPEC ONLY: it pins what to build — concrete Rust/Dart signatures, the TunnelPlatform contract impl, FFI surface, the helper-daemon IPC, state machines, lifecycle, privilege model, memory budgets, error handling, edge cases, and test points tied to the §11.4.169 closed test-type vocabulary. It does not write the shipping product. Sources cited inline by id — [03-client §N] = final/03-client-core-and-ui.md (the pass-1 client overview this deepens); [01-TXP §N] = final/v02-data-plane/transport-trait.md; [01-ORC §N] = final/v02-data-plane/orchestrator-and-state.md; [04_ARCH §N] = 04_VPN_CLD/HelixVPN-Architecture-Refined.md; [04_UI §N] = 04_VPN_CLD/HelixVPN-helix-ui-Flutter.md; [04_P0 §N] = 04_VPN_CLD/HelixVPN-Phase0-Spike.md; [SYN §N] = v09-research/_SYNTHESIS.md. [research-flutter_ffi] / [research-ios_android] = the FFI / native-shim pattern digests (referenced for the generic flutter_rust_bridge/UniFFI patterns doc 03 already grounds). Anything not grounded in that evidence base is tagged UNVERIFIED per constitution §11.4.6 — never fabricated.

## Table of contents

- 0. Position, ownership, and what makes Linux different

- 1. Non-negotiable Linux-shim invariants

- 2. The two data-path backends (kernel WG vs userspace tun)

- 3. Deployment topologies (in-process vs privileged helper daemon)

- 4. The TunnelPlatform contract — Linux impl

- 5. FFI surface consumed on Linux + the TUN handoff

- 6. The privileged helper daemon (helix-netd) + IPC

- 7. Privilege model — CAP_NET_ADMIN, rootless, polkit, setcap

- 8. systemd integration

- 9. The connector-daemon deployment (--mode=connector)

- 10. Shim lifecycle state machine

- 11. Kill-switch + DNS on Linux (nftables / resolved)

- 12. Memory / size / performance budgets

- 13. Error taxonomy & edge cases

- 14. Test points (§11.4.169)

- 15. Open decisions surfaced by this document

- 16. Cross-document contracts this document consumes/fixes

- Sources verified

## 0. Position, ownership, and what makes Linux different

The Linux shim is one row of the TunnelPlatform matrix [03-client §5, 04_ARCH §5.3]: configure the OS tunnel, hand packets to/from helix-core, report lifecycle — everything else is shared Dart [04_UI §6]. This document owns only the Linux instantiation of that contract; it does not re-specify the FFI surface (owned by [03-client §3]), the Transport trait ([01-TXP]), or the orchestrator/TunnelStatus ([01-ORC]) — it consumes them.

Linux is the reference / simplest shim platform [04_P0 §9, 03-client §5.4]: on desktop the core can drive the TUN in-process — there is no OS-managed extension process (unlike iOS Network Extension or Android VpnService), no mandatory IPC hop (unlike the Windows privileged service). That simplicity is exactly why Phase-0 gate G5 (flutter_rust_bridge drives the core from Dart) ships on Flutter-Linux first [04_P0 §9].

But Linux carries two structural complications the other shims do not:

| # | Linux-specific reality | Consequence |
| --- | --- | --- |
| L0a | Two real data-path backends exist — kernel WireGuard (netlink) and userspace /dev/net/tun — with different obfuscation reach (§2). | A backend-selection decision the core makes per active Transport, not a fixed choice. |
| L0b | CAP_NET_ADMIN is mandatory to create a TUN, program routes, and install the kill-switch, yet the Flutter GUI runs unprivileged. | A privilege seam (§7) — either file-caps on a helper or a polkit-gated helper daemon (§6), mirroring the Windows service model. |
| L0c | Linux is the connector's primary host — the headless --mode=connector daemon (§9) runs on Linux far more than the GUI does [04_ARCH §5.4, 01-ORC §9]. | This doc owns the daemon deployment too, not only the desktop GUI shim. |

Ownership boundary (CI1 [03-client §0.1]). The shim owns the OS tunnel lifecycle and privilege (create/configure/destroy the TUN, acquire CAP_NET_ADMIN, install/remove firewall rules). It does not own crypto, obfuscation, or status truth — helix-core is the only place WG + transport live (O2 [03-client §4.1]), and TunnelStatus is core-sourced (CI2). On Linux the core is linked in-process (desktop GUI / connector daemon) so the seam is a function-call boundary, not a socket — but the ownership split is identical to every other platform.

[MERMAID RENDER FAILED — block #1]

## 1. Non-negotiable Linux-shim invariants

Inherits the five TunnelPlatform obligations O1–O5 [03-client §4.1] and adds the Linux-specific ones below. A violation is a release blocker (§11.4 anti-bluff at the platform-shim layer; the shim is the only untyped seam — O5).

| # | Invariant | Why / mechanical consequence |
| --- | --- | --- |
| LX1 | The shim never crypts/obfuscates; it only creates the carrier device and hands the core a tun fd (userspace) or programs a kernel-WG peer (plain-udp). | O2 [03-client §4.1]; one core, no fork [01-TXP I4]. |
| LX2 | CAP_NET_ADMIN is acquired through exactly one sanctioned path (file-cap helper or polkit-gated helix-netd, §7) — never sudo from the GUI, never a setuid-root blob. | §11.4.161 rootless-first; §11.4.133 target-system safety. |
| LX3 | startTunnel is idempotent + permission-aware: if privilege acquisition is denied, emit permissionDenied (not error), leave no half-open tun / orphan route / dangling nft table (§11.4.14). | O1 [03-client §4.1]. |
| LX4 | On stopTunnel and on every abnormal exit path the shim restores a quiescent host: tun destroyed, routes withdrawn, nft kill-switch table flushed per the §8.6/§11 fail-closed rule (kill-switch stays closed on crash, opens only on user-stop). | O4 [03-client §4.1]; O-I11 [01-ORC §0.3]; trap/Drop/systemd ExecStopPost. |
| LX5 | The shim reports lifecycle as PlatformTunnelEvent (up/down/permissionDenied/revoked/error); it never synthesizes a TunnelStatus. Protection-state truth flows core→FFI→UI (CI2). | prevents "UI says connected while tunnel is down". |
| LX6 | Backend choice is determined by the active Transport::kind() (§2), never guessed: plain-udp MAY use kernel WG; any obfuscated carrier MUST use the userspace tun path. The shim asks the core which backend the resolved rung needs; it does not infer. | §11.4.6 no-guessing; §2.3. |
| LX7 | The connector daemon and the desktop GUI run the byte-identical core (--mode={client¦connector}), differing only in routing posture and privilege host (§9). | O-I13 [01-ORC §0.3], I4 [01-TXP]. |

## 2. The two data-path backends (kernel WG vs userspace tun)

The single most consequential Linux nano-detail. HelixVPN's obfuscation is pluggable beneath WireGuard [04_ARCH §2/§3, SYN §2]; helix-transport carriers (masque-h3, shadowsocks, udp-over-tcp, lwo, …) sit below the WG datagram [01-TXP §0]. This collides with how Linux kernel WireGuard works.

### 2.1 Kernel WireGuard (netlink) — plain-udp ONLY

Kernel WG owns its own UDP socket to the peer endpoint; it encrypts in the kernel and sends WG datagrams directly out a raw UDP port [04_ARCH §3 table, SYN §5]. It is the fast path (zero userspace copies, no per-packet context-switch). It is configured over netlink (the wireguard genl family)

- rtnetlink for addresses/routes — via the wireguard-control / defguard_wireguard_rs + rtnetlink crates (UNVERIFIED exact crate choice — a Phase-1 dependency decision; the netlink mechanism is grounded [04_ARCH §5.3]).

Hard limitation (FACT, not a guess — §11.4.6): kernel WG speaks only plain WireGuard-over-UDP. It cannot route its datagrams through a userspace MASQUE / Shadowsocks / TCP obfuscator. Therefore kernel WG is usable only when the active rung is plain-udp (the unrestricted-network baseline [01-TXP §4.1]).

plain-udp via kernel WG:
  [app traffic] → kernel route → wgX (kernel WG iface) → kernel encrypts
       → kernel UDP :51820 → peer        (helix-core only PROGRAMS the peer; no per-packet pump)

In this mode helix-core's data loops (01-ORC §3) do not pump packets — the kernel does the datapath. The orchestrator still owns control: reconcile, kill-switch, DNS, status. It programs the kernel WG device (peer pubkey, endpoint, AllowedIPs) and reads last_handshake_time / rx/tx counters via netlink to drive Handshaking → Connected and PeerTimeout (§10, §13).

### 2.2 Userspace /dev/net/tun — every obfuscated carrier

When the active rung is any obfuscating transport, the datapath MUST be userspace: the shim opens /dev/net/tun, ioctl(TUNSETIFF, IFF_TUN|IFF_NO_PI) to create tunX, programs addresses/routes/MTU on it, and hands the fd to the core via attach_tun(fd) [03-client §3.1, 01-ORC §2.1]. The core's three loops [01-ORC §3] then run: plaintext IP ← tun → helix-wg (boringtun userspace) encrypt → helix-transport obfuscate → OS socket; and the reverse.

masque-h3 (or any obfs) via userspace tun:
  [app traffic] → kernel route → tunX (/dev/net/tun) → read by helix-core loop A
       → helix-wg encrypt → helix-transport (masque-h3) → QUIC/H3 :443 → gateway

This is the path that the iOS NE / Android VpnService shims use universally (they have no kernel-WG option) [03-client §5.1/§5.2]; on Linux it is the obfuscated path and the fallback path when kernel WG is unavailable (no wireguard module, missing CAP, container without the genl family) [04_ARCH §5.3 "boringtun userspace only as fallback"].

### 2.3 Backend selection (the core decides, the shim obeys — LX6)

[MERMAID RENDER FAILED — block #2]

Selection rule (closed, §11.4.6 — no guessing):

- The core's ladder resolves the rung → Transport::kind() [01-TXP §6.3].

- kind() == "plain-udp" and the host advertised kernel_wg = true at startTunnel and KernelPathPolicy != ForceUserspace ⇒ KernelWg.

- Otherwise ⇒ UserspaceTun (the universal path; always available where /dev/net/tun + CAP_NET_ADMIN exist).

- A ladder escalation that changes kind() across the kernel/userspace boundary (e.g. plain-udp blocked → escalate to masque-h3) is a backend switch: the orchestrator's atomic carrier swap [01-ORC §7.3] is paired with a shim-side carrier-device swap — tear the kernel wgX, create the userspace tunX, attach_tun(fd), re-program routes/DNS/kill-switch on the new device. The kill-switch stays closed across the swap (O-I11) so no leak window.

Decision D-LXN-1 (surfaced, §11.4.66). Whether to ship the kernel-WG fast path in Phase 1 at all, or default every Linux session to the userspace tun for one uniform datapath (simpler, slightly slower for the plain-udp case). Option A (ship both): best plain-udp throughput on cooperative networks. Option B (userspace-only Phase 1): one datapath, no backend-switch complexity, matches iOS/Android exactly — defer kernel WG to Phase 2 as a perf optimisation. Recommendation: B for Phase 1 (kill-switch/backend-switch correctness > peak plain-udp throughput; the userspace path is the one every obfuscated carrier needs anyway), promote A in Phase 2 behind a measured BENCH gain. See §15.

## 3. Deployment topologies (in-process vs privileged helper daemon)

Two ways the privileged datapath is reached from the unprivileged Flutter GUI [04_ARCH §5.3, 03-client §5.4]. Both implement the same Dart TunnelPlatform (§4); they differ only in where CAP_NET_ADMIN lives.

| Topology | Core runs | Privilege held by | When | Mirrors |
| --- | --- | --- | --- | --- |
| T-A in-process | inside the Flutter process via frb | the Flutter process itself (file-caps on the launcher, §7.2) — or the app was launched by a service manager with caps | dev / single-user appliance / CLI; Phase-0 G5 reference [04_P0 §9] | (none — Linux-only simplicity) |
| T-B helper daemon | inside a privileged helix-netd system service (§6); Flutter is unprivileged | helix-netd (systemd AmbientCapabilities=CAP_NET_ADMIN, polkit-gated) | desktop GUI on a multi-user workstation — recommended for shipped desktop | the Windows privileged-service + named-pipe model [03-client §5.3] |

Why T-B for the shipped desktop GUI. Giving the whole Flutter+Skia process CAP_NET_ADMIN (T-A) widens the privileged attack surface to all of Dart + the UI toolkit. T-B confines privilege to a small audited Rust daemon (§6) and keeps the GUI unprivileged — the same reasoning that puts the core in a Windows service [03-client §5.3]. T-A remains the Phase-0 spike and the CLI / headless appliance topology where there is no separate GUI process to de-privilege [04_P0 §9].

In T-A the helix-core link is the frb in-process bridge [research-flutter_ffi; 03-client §3]; attach_tun(fd) is a direct call. In T-B the core lives in helix-netd; the Dart LinuxTunnelPlatform marshals startTunnel/stopTunnel over a Unix-domain socket (§6) and the status stream crosses the socket too — the GUI's frb binding subscribes to a relayed stream (§6.4), preserving CI2 (UI is a pure function of the core's status stream).

[MERMAID RENDER FAILED — block #3]

## 4. The TunnelPlatform contract — Linux impl

The Dart contract is fixed by [03-client §4] (one MethodChannel helixvpn/tunnel + one EventChannel helixvpn/tunnel/events, startTunnel/stopTunnel/events, TunnelConfig, PlatformTunnelEvent). On Linux the "channel" is not a Flutter platform channel to native code — there is no Kotlin/Swift layer — it is either an in-process frb call (T-A) or a UDS RPC (T-B). The Dart class is identical regardless:

// helix_core_ffi/lib/src/linux/linux_tunnel_platform.dart
// Implements the shared TunnelPlatform contract [03-client §4].
final class LinuxTunnelPlatform implements TunnelPlatform {
  LinuxTunnelPlatform(this._core, this._priv);     // _priv = InProcPriv (T-A) | NetdClient (T-B)
  final HelixCore _core;                            // frb-generated core handle [03-client §3.2]
  final PrivilegedTunnelOps _priv;                  // §4.1 — the only privileged seam

  @override
  Future<void> startTunnel(TunnelConfig cfg) async {
    // 1. ask the privileged plane to acquire CAP_NET_ADMIN + decide backend (§2.3)
    final backend = await _priv.acquireAndSelectBackend(cfg);   // throws PermissionDenied → §10
    // 2. create the carrier device under privilege; for userspace get the tun fd
    final created = await _priv.createCarrier(cfg, backend);     // KernelWgHandle | TunFd
    // 3. start the core; userspace path hands it the fd (CI1: core never opens the tun)
    await _core.start(transport: 'auto', mapPathOrSession: cfg.sessionOrMapToken);
    if (created is TunFd) { await _core.attachTun(created.fd); } // §5
    // 4. lifecycle event (NOT status — LX5); status arrives via _core.statusStream()
    _emit(PlatformTunnelEvent(PlatformTunnelEventKind.up));
  }

  @override
  Future<void> stopTunnel() async {
    await _core.detachTun();                         // userspace path; no-op on kernel path
    await _core.stop();
    await _priv.destroyCarrier();                    // tear tun/wgX, withdraw routes (LX4)
    await _priv.revertKillSwitch(userInitiated: true); // §11 — the ONLY open path (§8.6)
    _emit(PlatformTunnelEvent(PlatformTunnelEventKind.down));
  }

  @override
  Stream<PlatformTunnelEvent> events() => _events.stream;  // up/down/permissionDenied/revoked/error
}

### 4.1 PrivilegedTunnelOps — the privileged seam (the only Linux-specific surface)

// The abstraction that hides T-A (in-proc) vs T-B (helper daemon) from the rest of the app.
abstract interface class PrivilegedTunnelOps {
  /// Acquire CAP_NET_ADMIN (file-cap path: already held; helper path: polkit auth) and
  /// resolve the data-path backend for the ladder's chosen rung (§2.3). Throws
  /// PermissionDenied (→ permissionDenied event, LX3) if privilege is refused.
  Future<TunBackend> acquireAndSelectBackend(TunnelConfig cfg);

  /// Create the carrier device under privilege. KernelWg → program wgX peer (no fd);
  /// UserspaceTun → open /dev/net/tun, ioctl(TUNSETIFF), program addrs/routes/MTU, return fd.
  Future<CarrierHandle> createCarrier(TunnelConfig cfg, TunBackend backend);

  /// Switch backend mid-session on a ladder escalation crossing the kernel/userspace
  /// boundary (§2.3 swap); kill-switch stays closed across the swap (O-I11).
  Future<CarrierHandle> switchCarrier(TunBackend to, TunnelConfig cfg);

  Future<void> destroyCarrier();                 // idempotent (LX4)
  Future<void> applyKillSwitch(KillSwitchSpec s);// §11
  Future<void> revertKillSwitch({required bool userInitiated}); // §8.6 — open only if userInitiated
}

enum TunBackend { kernelWg, userspaceTun }
sealed class CarrierHandle {}
final class KernelWgHandle extends CarrierHandle { final String iface; KernelWgHandle(this.iface); }
final class TunFd          extends CarrierHandle { final int fd; final String iface; TunFd(this.fd, this.iface); }

TunnelConfig is unchanged from [03-client §4]: overlayIp, routes, dnsServers, splitExcludeApps (per-route bypass on Linux — see §11), mtu (1280 default over MASQUE, 1420 plain WG [03-client §4]), sessionOrMapToken.

## 5. FFI surface consumed on Linux + the TUN handoff

The Linux shim consumes the frozen FFI surface [03-client §3.1] — it adds no new FFI verbs except the tun-fd handoff that already exists for Android (attach_tun/detach_tun) [03-client §3.1, 01-ORC §2.1]. Reproduced here for the Linux-relevant subset (consistency anchor — the canonical definition lives in [03-client §3.1]):

// helix-ffi/src/api.rs (subset consumed on Linux) — canonical surface: [03-client §3.1]
pub async fn start(cfg: ClientConfig) -> anyhow::Result<()>;   // ClientConfig{ map_path_or_session, transport, mode }
pub async fn stop()  -> anyhow::Result<()>;
pub fn status_stream(sink: StreamSink<TunnelStatus>);          // §5.1
pub fn attach_tun(fd: i32) -> anyhow::Result<()>;              // userspace path: hand the /dev/net/tun fd
pub fn detach_tun() -> anyhow::Result<()>;                     // userspace path teardown; no-op on kernel path

### 5.1 TunnelStatus — consistency with the data plane (LX5)

The canonical data-plane status enum is the 5-variant TunnelStatus emitted by the orchestrator over tokio::broadcast [01-ORC §4.1]:

// CANONICAL — [01-ORC §4.1]. The Linux shim NEVER constructs this; it is core-sourced.
pub enum TunnelStatus {
    Connecting,
    Handshaking,
    Connected { transport: String, rtt_ms: u32 },   // transport = Transport::kind() (e.g. "plain-udp","masque-h3")
    Reconnecting,
    Down { reason: String },                          // reason prefix vocabulary: [01-ORC §4.4]
}

The FFI mirror [03-client §3.1] is a UI superset of the same enum — it adds Disconnected (the clean-idle projection of pre-start/stopped), a path field on Connected ("direct"|"relay", Phase 2 [03-client §3.1]), and Danger{kind} (leak/kill-switch-tripped, a UI projection driven by the §11 detector). Per [01-ORC §5.1], Idle/ShuttingDown map to Disconnected/Down at the FFI; Danger is not an orchestrator state — it is a UI overlay derived from a leak signal. The Linux shim emits none of these; it emits PlatformTunnelEvent only (LX5), and the orchestrator's broadcast reaches Dart over frb (T-A) or the relayed UDS stream (T-B, §6.4). This keeps the FFI surface byte-consistent with [01-ORC §4.1] and [03-client §3.1]; a drift FAILs the FFI contract test [03-client §12].

### 5.2 The userspace tun-fd lifecycle (the one Linux-specific FFI flow)

[MERMAID RENDER FAILED — block #4]

fd ownership. After attach_tun(fd) the core owns the fd for read/write; the shim retains the fd only to close() it on teardown after detach_tun() returns (double-close guard: detach_tun makes the core drop its read loop and relinquish the fd; the shim then closes — never the reverse). IFF_NO_PI (no 4-byte packet-info prefix) is mandatory so the core's loop A reads raw IP packets [01-ORC §3.1] (UNVERIFIED: exact flags confirmed against the running kernel in the Phase-0 IT rig, not assumed).

## 6. The privileged helper daemon (helix-netd) + IPC

T-B's privileged plane. A small Rust daemon that is the privileged carrier manager + core host; the unprivileged Flutter GUI is a thin client. Mirrors the Windows HelixTunnelSvc + named-pipe design [03-client §5.3] with a Unix-domain socket instead of a named pipe.

### 6.1 Surface (the same TunnelPlatform verbs, over UDS)

// helix-netd/src/ipc.rs — length-prefixed CBOR (or protobuf) frames over a SOCK_SEQPACKET UDS.
// Socket: /run/helix/netd.sock  (systemd socket-activated, §8). 0660 root:helix-netd group.
pub enum NetdRequest {
    StartTunnel { cfg: WireTunnelConfig },     // mirrors Dart TunnelConfig [03-client §4]
    StopTunnel,
    SetPin     { kind: Option<String> },        // manual transport pin [01-ORC §2.1 set_pin]
    ApplyMap   { map: WireRouteMap },           // Phase 0 file-watch stand-in is daemon-side; Phase 1 stream
    Subscribe,                                  // opens the status relay (§6.4)
}
pub enum NetdResponse {
    Ok,
    PermissionDenied { detail: String },        // → permissionDenied event (LX3)
    Error { detail: String },                   // honest reason (§11.4.6), never a secret (§11.4.10)
    Status(WireTunnelStatus),                    // streamed after Subscribe (§6.4)
}

### 6.2 Authn / authz of the IPC

- The UDS is 0660 root:helix-netd; only members of the helix-netd group (the GUI's runtime user, added at install) may connect — coarse gate.

- Per-action authorization is polkit (§7.3): StartTunnel/StopTunnel trigger the polkit action digital.vasic.helixvpn.manage-tunnel; the daemon calls polkit with the caller's SO_PEERCRED (pid/uid) to authorize, so an interactive auth prompt is shown for a non-allowlisted user (auth_admin_keep) and silently allowed for an allowlisted desktop session (UNVERIFIED: exact polkit rule shipped — a Phase-1 packaging decision).

- The daemon never accepts a WG private key or a SecretBytes over the IPC in the clear beyond what the wire config already carries (§11.4.10); secrets in WireTunnelConfig are redacted in every daemon log (§13).

### 6.3 Single-owner discipline

helix-netd serves one active tunnel at a time (one carrier device, one core). A second StartTunnel while a tunnel is up is rejected Error{busy} (idempotency: a StartTunnel with the same cfg fingerprint is a no-op Ok — LX3). This is the §11.4.119 single-resource-owner rule applied to the host's tun device.

### 6.4 Status relay (preserving CI2 across the socket)

[MERMAID RENDER FAILED — block #5]

The relay applies the same receiver contract as the FFI [01-ORC §4.6]: Lagged → skip-to-latest (never an error), Closed → daemon gone → GUI shows Down{reason:"helper-unavailable"} and offers reconnect. Coalescing ([01-ORC §4.3]) happens core-side before the relay, so the socket carries only rare state-change frames.

## 7. Privilege model — CAP_NET_ADMIN, rootless, polkit, setcap

### 7.1 What needs privilege (FACT — §11.4.6)

| Operation | Capability required | Used by |
| --- | --- | --- |
| open("/dev/net/tun") + ioctl(TUNSETIFF) (create tun) | CAP_NET_ADMIN | userspace backend (§2.2) |
| Create/configure a kernel wireguard netlink device | CAP_NET_ADMIN | kernel backend (§2.1) |
| rtnetlink addr/route/MTU programming | CAP_NET_ADMIN | both backends |
| nftables kill-switch table install/flush | CAP_NET_ADMIN | §11 |
| Bind a low transport port (none — carriers bind ephemeral/:443-dest, not listen) | none (client dials outbound, SYN §1) | — |

Honest boundary (§11.4.6). A fully unprivileged (zero-CAP) system-wide Linux tunnel is not possible — creating a TUN and programming routes both require CAP_NET_ADMIN; this is a kernel constraint, not a design choice (cf. §11.4.112 structural-impossibility class). UNVERIFIED: some distros allow a pre-created persistent tun owned by a group (tunctl/ip tuntap … user) that an unprivileged process may then open, but route/firewall programming still needs CAP_NET_ADMIN — so the privilege cannot be fully eliminated, only confined (T-B). The §11.4.161 "rootless" mandate is satisfied by never escalating to full root — CAP_NET_ADMIN ≠ root, and the daemon drops every other capability (§8.2).

### 7.2 setcap / file-capability path (T-A, and the helper binary in T-B)

# install-time (packaging), the ONLY sanctioned grant (LX2):
setcap 'cap_net_admin,cap_net_raw=ep' /usr/lib/helixvpn/helix-netd     # T-B helper
# (T-A appliance/CLI:)  setcap 'cap_net_admin=ep' /usr/bin/helixvpn-cli

cap_net_raw is added only if a carrier needs raw sockets (UNVERIFIED — none of the §2 carriers does today; included defensively for ICMP-based path probing, removed if unused per §11.4.124 dead-capability discipline). The GUI binary never carries file-caps in T-B (LX2).

### 7.3 polkit (T-B per-action gate)

A polkit action file digital.vasic.helixvpn.manage-tunnel (allow_active = auth_admin_keep by default; a desktop install MAY relax to yes for the console user via a shipped rule). helix-netd authorizes each StartTunnel/ StopTunnel against the caller's SO_PEERCRED. Denial → PermissionDenied → permissionDenied event (LX3) — never an error (clean UX, [03-client O1]).

### 7.4 Rootless containerized path (connector, §11.4.161)

For the connector daemon shipped as a rootless Podman container (§9.4): the container is granted --cap-add NET_ADMIN and --device /dev/net/tun and runs as a non-root UID. UNVERIFIED: rootless Podman + /dev/net/tun + NET_ADMIN requires the host tun module loaded and the rootless user's netns to permit it — verified in the §11.4.76 containers-submodule on-demand IT rig, never assumed. The userspace tun backend (§2.2) is mandatory inside a container (kernel WG netlink to the host wg subsystem is generally not available to a rootless container netns — UNVERIFIED, measured in IT).

## 8. systemd integration

### 8.1 Desktop helper units (T-B)

# /usr/lib/systemd/system/helix-netd.socket  — socket activation (daemon idle until first connect)
[Socket]
ListenStream=/run/helix/netd.sock
SocketMode=0660
SocketUser=root
SocketGroup=helix-netd
[Install]
WantedBy=sockets.target

# /usr/lib/systemd/system/helix-netd.service
[Service]
Type=notify                         # sd_notify READY=1 once the IPC loop is serving (§8.3)
ExecStart=/usr/lib/helixvpn/helix-netd
AmbientCapabilities=CAP_NET_ADMIN   # the ONLY cap granted (LX2 / §7.1); NOT root
CapabilityBoundingSet=CAP_NET_ADMIN # nothing else is even reachable
NoNewPrivileges=yes
ProtectSystem=strict
ProtectHome=yes
PrivateTmp=yes
ProtectKernelModules=yes
ProtectControlGroups=yes
RestrictAddressFamilies=AF_UNIX AF_INET AF_INET6 AF_NETLINK   # UDS + sockets + netlink only
SystemCallFilter=@system-service
MemoryDenyWriteExecute=yes
ExecStopPost=/usr/lib/helixvpn/helix-netd --flush-killswitch-on-crash   # LX4 / O-I11

ProtectSystem=strict etc. are §11.4.133 target-system-safety hardening; the daemon keeps only CAP_NET_ADMIN (§7.1). RestrictAddressFamilies includes AF_NETLINK (kernel-WG/route programming) and AF_INET/6 (the transport carriers dial outbound). ExecStopPost enforces fail-closed kill-switch on an abnormal stop (O-I11 [01-ORC §0.3]).

### 8.2 sd_notify lifecycle (the daemon ↔ systemd contract)

// helix-netd readiness + watchdog (Type=notify, WatchdogSec= set in §9 connector unit).
sd_notify(false, "READY=1\nSTATUS=serving on /run/helix/netd.sock");
// per loop: sd_notify(false, &format!("WATCHDOG=1"));               // liveness ping
// on reconfigure: sd_notify(false, "RELOADING=1"); … "READY=1";
// on stop:        sd_notify(false, "STOPPING=1\nSTATUS=tearing down tunnel");

### 8.3 Why socket activation

The desktop helper is idle (not running) until the GUI first connects — zero resident cost when no tunnel is wanted. systemd starts helix-netd on the first UDS connection and (optionally) stops it after an idle timeout post-stopTunnel. For the connector (§9) socket activation is off — it is a persistent network appliance that must reconnect autonomously (fail-static, I3 [01-ORC §0.3]).

## 9. The connector-daemon deployment (--mode=connector)

The connector is the same helix-core, Mode::Connector — byte-identical loops, status stream, reconciler, transport ladder; only the routing posture differs (O-I13 [01-ORC §9], I4 [01-TXP]). Linux is its primary host [04_ARCH §5.4]. It is headless (no Flutter UI required; the Connector flavor GUI is optional config surface [03-client §6, D-CLIENT-4]).

### 9.1 What differs from the GUI client

| Aspect | GUI client | Connector daemon |
| --- | --- | --- |
| Topology | T-A or T-B (§3) | standalone privileged service (no GUI; core in the daemon process) |
| Carrier device | tun capturing the device default route | LAN iface + tun; forwards decapsulated packets into the served CIDR + NAT (no default-route capture) [01-ORC §9.1] |
| Kill-switch | Strict/Permissive (privacy) | typically Off — it is an appliance, not a privacy client (config knob, [01-ORC §9.4]) |
| Inbound | n/a | outbound-only — dials the gateway, never listens (founding no-port-forward constraint [SYN §1]) |
| Verdict map | n/a | installs the compiled policy as an nftables/eBPF verdict map, re-installed <1s on reconcile, default-deny fail-closed [01-ORC §9.2/§9.3] |
| Lifecycle host | GUI process / helper | systemd Type=notify service or rootless Podman quadlet (§9.4) |

### 9.2 The connector data path (loop B branch)

[MERMAID RENDER FAILED — block #6]

The connector runs loop A driven by the LAN iface (not a TUN) and loop B forwarding into the LAN — TunSpec abstracts both so the loop bodies are unchanged [01-ORC §9.2]. CAP_NET_ADMIN is required for the tun, the LAN-forward NAT, and the nftables verdict map (§7.1).

### 9.3 systemd unit (connector)

# /usr/lib/systemd/system/helix-connectord.service
[Unit]
Description=Helix VPN Connector (outbound-only network agent)
After=network-online.target
Wants=network-online.target
[Service]
Type=notify
ExecStart=/usr/lib/helixvpn/helix-connectord --mode=connector --config /etc/helixvpn/connector.toml
AmbientCapabilities=CAP_NET_ADMIN
CapabilityBoundingSet=CAP_NET_ADMIN
NoNewPrivileges=yes
WatchdogSec=30                       # §8.2 sd_notify WATCHDOG=1 every <15s; restart on hang
Restart=on-failure
RestartSec=2                         # bounded; complements the core's own backoff [01-ORC §7]
ProtectSystem=strict
ProtectHome=yes
PrivateTmp=yes
RestrictAddressFamilies=AF_UNIX AF_INET AF_INET6 AF_NETLINK
SystemCallFilter=@system-service
[Install]
WantedBy=multi-user.target

Restart=on-failure + WatchdogSec give the appliance autonomous recovery on top of the core's in-process reconnect ladder [01-ORC §7] — the systemd restart is the last-resort supervisor (E10 [01-ORC §12]: a panicked core is restarted; start() resets the ladder). The credential file /etc/helixvpn/connector.toml and any enrollment secret are 0600 root-owned and git-ignored (§11.4.10/.30) — never in the unit, never logged.

### 9.4 Rootless Podman quadlet (§11.4.76 / §11.4.161)

Per §11.4.76 all containerized deploy goes through the vasic-digital/containers submodule; the connector ships as an OCI image run by a Podman quadlet (.container unit managed by systemd, rootless) [04_ARCH §4, SYN §2]:

# ~/.config/containers/systemd/helix-connectord.container  (rootless quadlet)
[Container]
Image=ghcr.io/vasic-digital/helix-connectord:helixvpn-<version>   # §11.4.151 prefix
AddCapability=NET_ADMIN              # ONLY this; rootless, non-root UID inside (§11.4.161)
AddDevice=/dev/net/tun
Exec=--mode=connector --config /etc/helixvpn/connector.toml
Notify=true                          # quadlet → Type=notify passthrough
[Service]
Restart=on-failure
[Install]
WantedBy=default.target

Inside the container the userspace tun backend is mandatory (§7.4); the container boots its integration infra on-demand via the containers submodule (§11.4.76 on-demand-infra invariant) for the IT test rig (§14).

## 10. Shim lifecycle state machine

The shim's own lifecycle (distinct from the core's OrchState [01-ORC §5]). It emits PlatformTunnelEvent (LX5), never TunnelStatus.

[MERMAID RENDER FAILED — block #7]

State→event map (the only events crossing to the UI per [03-client §4]):

| Shim state reached | PlatformTunnelEventKind emitted | Notes |
| --- | --- | --- |
| Up | up | UI still waits for the core Connected before painting green (CI2) |
| PermissionDenied | permissionDenied | distinct from error (LX3) — UI offers "grant access" |
| CarrierError | error (+ honest detail) | tun/netlink failure; no half-open device left (LX4) |
| Revoked | revoked | admin/out-of-band kill (O3); core also emits Down{auth-failed} |
| Cleanup→Idle | down | host quiescent; kill-switch closed unless user-initiated stop (§8.6) |

## 11. Kill-switch + DNS on Linux (nftables / resolved)

The kill-switch and DNS-leak protection are owned by the core state machine [01-ORC §8], applied by the privileged plane on Linux. The shim never hand-edits rules independently — it executes the core's side-effect commands (O-I10 [01-ORC §0.3]).

### 11.1 nftables kill-switch

# installed on Idle→Connecting (Strict mode) [01-ORC §8.2]; flushed only on user-stop (§8.6)
table inet helix_killswitch {
  chain output {
    type filter hook output priority 0; policy drop;       # default-DROP egress (fail-closed, O-I11)
    oifname "helix0" accept                                  # the tun/wg carrier device
    ip daddr <gateway_endpoints> udp dport 443 accept        # allow_gateway_endpoints [01-ORC §8.2]
    ip daddr <gateway_endpoints> tcp dport 443 accept        # masque/uot dial
    meta l4proto { icmp, icmpv6 } accept                     # ND/DHCP link bring-up
    ip daddr <rfc1918> accept                                # allow_lan (config; printers etc.)
  }
}

<gateway_endpoints> = KillSwitchConfig.allow_gateway_endpoints [01-ORC §8.2] — the carrier MUST always reach :443. The table is installed closed before the first handshake (Strict) so there is no pre-connect leak, and stays installed on crash (O-I11) — the systemd ExecStopPost (§8.1) only flushes on a clean user stop (§8.6). splitExcludeApps [03-client §4] maps on Linux to per-cgroup / per-fwmark accept rules (UNVERIFIED exact mechanism — Phase-2 split-tunnel item; the cgroup-net-cls / nftables meta cgroup path is the design reference).

### 11.2 DNS (the DnsApplyMethod matrix [01-ORC §8.3])

| Host DNS stack | DnsApplyMethod | Mechanism |
| --- | --- | --- |
| systemd-resolved present | systemd-resolved | resolvectl dns helix0 <tunnelDns> + resolvectl domain helix0 '~.' (route all) |
| resolvconf present | resolvconf | resolvconf -a helix0 with the tunnel servers |
| neither (static) | /etc/resolv.conf | atomic rewrite + restore-on-stop (backup the original) |

block_plaintext_53 [01-ORC §8.3] adds nft rules dropping any off-tunnel :53 (UDP+TCP) when Connected — closing the classic DNS leak. The block stays on Reconnecting/Down (fail-closed) and reverts only on user-stop (§8.6, [01-ORC §8.4]). A DNS-apply failure at start is host-fatal → core emits Down{host-fatal} [01-ORC §8.2] — the shim never proceeds with an unverified DNS posture (§11.4.6).

## 12. Memory / size / performance budgets

Linux has no iOS-NE-class hard ceiling (CI4 [03-client §0.1] is iOS-only) — but the connector appliance / embedded host and the §11.4.161 rootless container warrant a real budget. Captured in the Phase-0 MEM/BENCH evidence, never asserted (§11.4.6/.123) [04_P0 §8].

| Budget | Target | Basis |
| --- | --- | --- |
| Kernel-WG backend userspace RSS | near-zero datapath (kernel owns packets); core resident = control only | §2.1 — fast path |
| Userspace-tun backend core RSS | low-tens MB steady-state (boringtun + transport buffers); no per-datagram heap growth | shares the helix-core budget [03-client §10]; reused scratch buffers [01-ORC §3.1] |
| helix-netd daemon RSS (idle, socket-activated) | 0 (not running) until first connect (§8.3) | socket activation |
| helix-netd RSS (active, T-B) | core RSS + a thin IPC relay (single-digit MB overhead) | §6 |
| Connector daemon RSS | low-tens MB; bounded by peers × Tunn + verdict map (no per-conn state, I5) | [01-ORC §13.2] |
| Through-tunnel throughput (plain-udp, kernel WG) | ≥ 80% bare link (G1) | [04_P0 §8]; kernel datapath ≈ native WG |
| Through-tunnel throughput (userspace tun, masque-h3) | ≥ 50% of plain-udp (G2); double-crypto/CC tax acknowledged | [01-TXP §11], [04_P0 §8] |
| Roam / iface-flap recovery | < 3 s | [01-ORC §7.5]; netlink iface watcher posts RoamDetected |
| Backend switch (kernel→userspace) latency | bounded; kill-switch closed throughout (no leak window) | §2.3, O-I11 |

The Linux iface watcher feeding RoamDetected [01-ORC §7.5] is netlink (RTM_NEWADDR/RTM_DELADDR on the rtnetlink route socket) — the shim's only ongoing background duty besides the carrier pump.

## 13. Error taxonomy & edge cases

Shim-layer errors map to PlatformTunnelEvent (§10); core errors flow through TunnelStatus ([01-ORC §10]). The shim's honest detail strings (§11.4.6) never carry a secret (§11.4.10).

| Code | Cause | Event / mapping |
| --- | --- | --- |
| LX-E1 | polkit / file-cap refused | permissionDenied (NOT error, LX3) |
| LX-E2 | /dev/net/tun absent (no tun module) | error{detail:"tun-module-missing"}; suggest modprobe tun (CLI) |
| LX-E3 | ioctl(TUNSETIFF) / netlink addr/route failed | error + full host cleanup (LX4); no orphan iface |
| LX-E4 | kernel wireguard genl family absent (kernel backend) | fall back to userspace tun (§2.3 rule 3), not an error |
| LX-E5 | nft kill-switch install failed | core → Down{host-fatal} [01-ORC §8.2]; shim emits error |
| LX-E6 | helix-netd UDS unreachable (T-B) | error{detail:"helper-unavailable"}; GUI offers retry/install |

Edge cases (each → a §14 test point):

| # | Edge case | Required behaviour |
| --- | --- | --- |
| E-LX1 | startTunnel called twice (same cfg) | idempotent no-op Ok (LX3); different cfg while up → Error{busy} (§6.3) |
| E-LX2 | privilege granted but tun create fails | full cleanup, no half-open device (LX4); error |
| E-LX3 | core crashes while shim holds the tun fd | T-A: process dies, kill-switch rules persist (O-I11), systemd/supervisor restarts; T-B: daemon ExecStopPost flush-on-crash leaves closed rules |
| E-LX4 | detach_tun then shim closes fd, but a core read loop lingered | ordering guard (§5.2): core drops the loop on detach_tun before shim close(); double-close impossible |
| E-LX5 | ladder escalates plain-udp(kernel)→masque-h3(userspace) mid-session | backend switch (§2.3): tear wgX, create tunX, attach_tun, re-program routes/DNS/KS; kill-switch stays closed across the swap (O-I11); recovery within the [01-ORC §7] budget |
| E-LX6 | host has no systemd (e.g. minimal container/Alpine-OpenRC) | T-A path only; helper-daemon socket-activation degrades to a plain spawned daemon; UNVERIFIED non-systemd init support is a Phase-2 packaging item |
| E-LX7 | device.revoked arrives out-of-band (admin kills the connector) | core → Down{auth-failed}; shim emits revoked (O3); kill-switch stays closed (connector Off → traffic just stops forwarding) |
| E-LX8 | rootless container lacks NET_ADMIN | startTunnel → permissionDenied with detail:"container-missing-net-admin"; documented remediation (§7.4) — never a fake success |

## 14. Test points (§11.4.169)

Every Linux-shim workable item declares its required test types from the §11.4.169 closed vocabulary [06-phase0-spike-wbs §0.4]; the ONLY permitted absence of a warranted type is an honest §11.4.3 SKIP-with-reason, never a silent gap. Four-layer enforcement per §11.4.4(b) on every closure. Every PASS ships rock-solid captured evidence (§11.4.5/.69/.107) — config-only / grep-only PASS forbidden.

| Test id | §11.4.169 types | What it proves | Rig / captured evidence |
| --- | --- | --- | --- |
| LX-1 | UT | PrivilegedTunnelOps backend-selection table (§2.3) is total + correct for every kind() × kernel_wg× policy | table-driven Rust/Dart unit test; mocks allowed (unit only, §11.4.27) |
| LX-2 | UT | nft kill-switch ruleset render is exact for Strict/Permissive/Off + allow_lan/allow_gateway | golden nft-script diff |
| LX-3 | IT | userspace path: open /dev/net/tun, attach_tun(fd), loop A/B move real WG datagrams; curl http://10.10.0.20/ over plain-udp and masque-h3 | netns + edge + connector booted via containers submodule (§11.4.76); tcpdump + curl 200 |
| LX-4 | IT | kernel-WG path: program wgX via netlink, last_handshake_time drives Connected; curl over plain-udp | netns + kernel wireguard; netlink dump + curl 200 |
| LX-5 | E2E+FA | full status arc Connecting→Handshaking→Connected{plain-udp,rtt} driven ONLY by the core stream on Flutter-Linux (G5) | window-scoped MP4 + vision/OCR verdict (§11.4.159) of StatusChip reading plain-udp · 23ms |
| LX-6 | E2E+FA | backend switch plain-udp(kernel)→masque-h3(userspace) on a DPI block; zero leak across the swap (E-LX5) | nft block plain WG mid-session; tcpdump underlay shows ZERO app packets during swap; ordered status trace (N=3 identical, §11.4.50) |
| LX-7 | SEC | kill-switch closes on drop — no plaintext egress while Reconnecting/Down; closed rules persist on simulated crash (E-LX3) | block gateway mid-Connected; tcpdump underlay = ZERO leaked packets; SIGKILL the core, assert nft table still present |
| LX-8 | SEC | DNS-leak guard: off-tunnel :53 dropped when Connected; system DNS restored on user-stop only (§11.2) | dig @off-tunnel blocked; resolvectl/resolv.conf capture before/after |
| LX-9 | SEC | privilege confinement: helix-netd holds only CAP_NET_ADMIN; GUI binary carries no file-caps (LX2); secrets absent from daemon logs (§11.4.10) | getpcaps/capsh dump; getcap on GUI; plant-a-secret grep-empty proof |
| LX-10 | E2E | polkit denial → permissionDenied (NOT error), no half-open tun (LX3/E-LX2) | deny the polkit action; assert event kind + ip tuntap show empty |
| LX-11 | INT | connector posture: gateway→connector→LAN forward + reverse, ACL-gated default-deny; verdict map re-install <1s on reconcile | netns LAN; authorized host reachable, unauthorized DROPPED+counted; pid unchanged on policy edit |
| LX-12 | SC | iface flap → RoamDetected (netlink) → re-dial → Connected <3s; process-kill mid-transfer → systemd restart recovers | netns flap + SIGKILL; status-trace timing CSV; recovery <3s |
| LX-13 | CONC+RACE | concurrent startTunnel/stopTunnel + backend switch under load drop zero datagrams; UDS relay no deadlock | concurrency harness; cargo test --features loom on the swap; relay log |
| LX-14 | MEM+BENCH | kernel-WG ≥80% bare link (G1); userspace masque ≥50% of plain (G2); core RSS bounded, no per-datagram heap growth | iperf3 via bench.sh CSV; /proc/<pid>/status RSS sample |
| LX-15 | LOAD | connector under 1/10/100 concurrent client sessions + handshake-flood: no fd leak, bounded mem, verdict map holds | iperf3 fan-in + handshake-flood CSV; ls /proc/<pid>/fd count stable |
| LX-16 | CH+HQA | a challenges/helix_qa bank entry scores PASS only on the captured LX-5/LX-6/LX-7 evidence (not config) | HelixQA autonomous session, evidence-gated (§11.4.27/.107); Challenge result.json |

Each gate ships a paired §1.1 meta-test mutation so it provably cannot bluff — e.g. flip the shim to close() the tun fd before detach_tun returns → LX-13 double-close guard FAILs; weaken the connector verdict gate → LX-11 FAILs; emit up before privilege is acquired → LX-10 FAILs; flush the kill-switch on crash → LX-7 FAILs.

## 15. Open decisions surfaced by this document

Per §11.4.6/§11.4.66 — options + recommendation, never silently resolved.

| # | Decision | Option A | Option B | Recommendation |
| --- | --- | --- | --- | --- |
| D-LXN-1 | Kernel-WG fast path in Phase 1? | ship both backends (best plain-udp throughput) | userspace-tun-only Phase 1 (one datapath, no swap complexity, matches iOS/Android) | B for Phase 1; promote A in Phase 2 behind a measured BENCH gain (§2.3). |
| D-LXN-2 | Desktop GUI privilege topology | T-A in-process (file-caps on the GUI) | T-B helper daemon helix-netd (polkit, confined caps) | T-B for the shipped GUI (small audited privileged surface, mirrors Windows); T-A for Phase-0 spike + CLI/appliance (§3). |
| D-LXN-3 | netlink crate for kernel WG + routes | defguard_wireguard_rs + rtnetlink | hand-rolled genl/rtnetlink | A (reuse a maintained crate) per §11.4.74 catalogue-first; UNVERIFIED maturity — confirm in Phase-1 spike before committing. |
| D-LXN-4 | Split-tunnel (splitExcludeApps) mechanism | cgroup-net-cls + fwmark + nft meta cgroup | network-namespace per app | A (Phase 2) — lower blast radius, no per-app netns; surfaced, not built in MVP (§11.1). |
| D-LXN-5 | Non-systemd init support (OpenRC/Alpine) | T-A spawned daemon only | full OpenRC service scripts | A for MVP, B as a Phase-2 packaging item (E-LX6); UNVERIFIED demand. |

## 16. Cross-document contracts this document consumes/fixes

| Contract | Source of truth | This doc's role |
| --- | --- | --- |
| FFI surface (start/stop/status_stream/attach_tun/detach_tun) | [03-client §3.1] | consumes unchanged; adds no FFI verb (§5) |
| TunnelStatus (5-variant canonical) | [01-ORC §4.1] | consumes; shim never constructs it (LX5); FFI mirror superset noted (§5.1) |
| TunnelPlatform channel contract (startTunnel/stopTunnel/events, TunnelConfig, PlatformTunnelEvent) | [03-client §4] | implements for Linux (§4); fixes the PrivilegedTunnelOps Linux seam |
| Transport::kind() → backend selection | [01-TXP §2/§6] | consumes to choose kernel-WG vs userspace tun (§2.3, LX6) |
| Kill-switch/DNS side-effect matrix | [01-ORC §8] | applies on Linux via nftables/resolved (§11) |
| Connector posture (Mode::Connector) | [01-ORC §9] | fixes the Linux daemon deployment (§9): systemd + rootless Podman quadlet |
| CAP_NET_ADMIN / rootless / containers | §11.4.76/.161/.133, [SYN §2] | fixes the privilege model (§7) + quadlet deploy (§9.4) |

## Sources verified

- final/03-client-core-and-ui.md [03-client] — §0.1 (CI1/CI2/CI4 invariants), §3 (FFI surface + TunnelStatus FFI mirror), §4 (TunnelPlatform contract, obligations O1–O5), §5.3 (Windows service/pipe parallel), §5.4 (Linux row), §6 (flavors), §10 (budgets), §12 (testing).

- final/v02-data-plane/orchestrator-and-state.md [01-ORC] — §0.3 (O-I10/11/13), §2.1 (attach_tun, Mode, TunSpec), §3 (three loops), §4.1 (canonical TunnelStatus), §4.4 (Down.reason), §5 (state machine), §7 (backoff/roam), §8 (kill-switch + DNS apply methods), §9 (connector posture/verdict map), §12 (edge cases), §13.2 (perf budgets).

- final/v02-data-plane/transport-trait.md [01-TXP] — §0/§2 (Transport, kind()), §4 (per-carrier dial: plain-udp vs obfuscated), §6 (ladder/dial), §11 (throughput budgets G1/G2).

- 04_VPN_CLD/HelixVPN-Architecture-Refined.md [04_ARCH] — §3 (WG crypto core + pluggable transports), §5.3 (per-platform shim matrix; Linux = kernel WG/tun, Rust), §5.4 (connector daemon), kernel-vs-userspace WG note, Podman quadlets.

- 04_VPN_CLD/HelixVPN-Phase0-Spike.md [04_P0] — §9 (G5 Flutter-Linux FFI demo; "on Linux the core can drive the TUN directly"), §8 (gates / recovery <3s).

- 04_VPN_CLD/HelixVPN-helix-ui-Flutter.md [04_UI] — §6 (shim contract: shim does only configure/handoff/lifecycle).

- v09-research/_SYNTHESIS.md [SYN] — §1 (outbound-only), §2 (rootless Podman, kernel-WG-fast-path/boringtun-fallback), §5 (client architecture), §9 (constitution bindings).

- 06-phase0-spike-wbs.md §0.4 — the §11.4.169 test-type code map (UT/IT/E2E/FA/ CH/HQA/LOAD/SEC/SC/CONC/RACE/MEM/BENCH).

- [research-flutter_ffi] — generic flutter_rust_bridge v2 in-process binding pattern (as grounded by [03-client §3]); [research-ios_android] — native-shim precedent (Linux in-process contrast). Items not in the read evidence base are tagged UNVERIFIED per §11.4.6 (kernel-WG crate choice, IFF_NO_PI flags, rootless-container tun reachability, polkit rule, non-systemd init, split-tunnel cgroup mechanism).

Constitution: §11.4.6 (no-guessing / UNVERIFIED tags), §11.4.10 (secret redaction), §11.4.66 (decisions = options + recommendation), §11.4.76 (containers submodule), §11.4.112 (structural-impossibility honesty — zero-CAP tunnel), §11.4.119 (single-resource-owner), §11.4.124 (dead-capability discipline), §11.4.133 (target-system safety / systemd hardening), §11.4.151/.155 (release/ recording prefixes), §11.4.159 (window-scoped vision-verified evidence), §11.4.161 (rootless), §11.4.169 (test-type coverage), §1.1 (paired mutations).

End of nano-detail specification — Linux shim (kernel WG / tun), Volume 4 (Clients). Pairs with [03-client §5.4] (the row this deepens), [01-ORC] (the core it hosts in-process), [01-TXP] (the carrier whose kind() drives backend selection), and the sibling shim docs (apple / android / windows / harmonyos / aurora). Surfaced decisions: D-LXN-1..5 (§15) — presented, none silently resolved (§11.4.66).